MATERIAL SAFETY DATA SHEET DATE PREPARED - MAY  1995

NFPA RAT Health: 3 Flammability: Reactivity: o Personal Protection: B

: BATTERY CLEANER & ACID LEAK DETECTOR

Section I

Page 1

Winzer Corporation

10560 Markison Rd. Dallas, Texas 75238 Telephone: 800-527-4126

BATTERY CLEANER & ACID DETECTOR

Part #: 891.4526081 MSDS#: 891.4526081.7

Section II - Hazardous Ingredients/Identity Information

Hazardous Components (Specific Chemical Identity, Common Name(s)

CAS #    OSHA PEL      ACGIH TLV ETHYLENE GLYCOL MONOBUTYL-

ETHER

25-SKIN

NA

NA

25-SKIN

NA

800   PPM

111-76-2 493-52-7 68476-86-8

METHYL RED  SODIUM  SALT

ISOBUTANE

STEL

DOT SHIPPING - Consumer Commodity    ORM-D

Section Ill-Physical/Chemical  Characteristics

Boiling Point-Range:  WIDE RANGE Vapor Pressure PSIG @  70F:   30-40

Vapor Density(Air=l)

Heavier than Air:   YES Solubility in Water:   COMPLETE

Specific Gravity(H20=l):  NA Melting Point-Liquid:   NA

Evaporation Rate:   SLOWER

(Water=l)

Appearance and Odor: YELLOW LIQUID-TURNS PINK IN-CONTACT W/ACID,  SOLVENT ODOR

Section IV-Fire and Explosion Hazard Data

Flash Point (Method Used): -40 F

TCC  Flammable Limits

Extinguishing Media

Use water fog, dry chemical or carbon dioxide

LEL : NA

UEL : NA

Special Fire Fighting Procedures

Fire fighters should wear NIOSH approved positive pressure self-

contained breathing apparatus. Aerosol cans may rupture when heated. Unusual Fire and Explosion Hazards

Heated cans may burst.

Section V - Reactivity Data	Page-2

Stability — Unstable:         Conditions to Avoid-: Heat, sparks, and open
Stable: Yes	: flames.

Incompatibility (Materials to Avoid): Strong acids or bases, oxidizing agents

Hazardous Decomposition or Byproducts: In fire:  water, carbon dioxide, carbon : monoxide, products of incomplete combustion.

Hazardous       May Occur:         Conditions to Avoid: None Polymerization  Will Not Occur: XX   :

Section VI - Health Hazard Data

Routes of Entry —         Inhalation?: Yes  Skin?: Yes    Ingestion?: Yes

Health Hazards (Acute and Chronic)—

May cause dizziness or narcosis in high vapor concentrations.  Will cause defat ting of skin.  Effects are reversible.  Long term exposure (years) to vapor may cause lung, liver, or kidney damage.  The solvents listed have been reported t, the central nervous system.

Carcinogenicity	NTP?         IARC Monographs?      OSHA Reg.?

: IARC cancer review indeterminant.

• •

Signs and Symptoms of Exposure-  Inhalation - Difficulty in breathing.
Skin -Redness	Ingestion- vomiting

Medical Conditions

Generally aggravated by Overexposure-

: None known.

Emergency and First Aid Procedures

Give oxygen-do not induce vomiting-gastric lavage-wash eyes and skin with

water.

: Call a physician immediately.

Section VII - Precautions for Safe handling and Use

Steps to be Taken in Case Material is Released or Spilled—

Use absorbent sweeping compound to soak up material.  Pour into container.  Dispose as hazardous waste.

Waste Disposal Method—

Dispose of waste in accordance with applicable Federal, State, and local requirements .

Precautions to be Taken in Handling and Storing-Keep away from heat, sparks or open flame.  Store at temperatures below 120F.

Other Precautions –

Avoid prolonged breathing of vapors.  Always use with adequate ventilation.

Section VIII  -  Control Measures

Respiratory Protection (Specific Type) –

Self contained breathing apparatus if above TLV limits are exceeded.

Ventilation  Local Exhaust:  Yes		Special:  None

Mechanical (General):  None		Other:  None

Protective Gloves – None required if spraying

Eye Protection – Safety glasses or goggles

Other Protective Clothing or Equipment – Long Sleeves and long pants or coveralls

Work/Hygiene Practices – Do not smoke while using.  Wash hands after use.